| results.csv 94.4% + latency, dataset.csv 99, confusion.png 100% (overfit note), task_results.csv above |  |  |
| --- | --- | --- |
| Thought · 1.4s Good — you already have the software side ready (arduino/arm.ino + src/06_serial_control.py). Hardware is just wiring + serial. 1. What you need: - Arduino Uno + USB - 4x SG90 + 4-DOF arm frame (or 6-DOF, we use 4) - 5V 2A supply for servos alone + jumpers - Laptop with your RoboGrip venv 2. Wire (critical: external power, not Arduino 5V): Servo signal: base→D9, shoulder→D6, elbow→D5, grip→D3 Servo + → external 5V+, Servo - → external GND + Arduino GND (common!) Arduino USB → laptop If arm jitters/resets = power issue, 99% of cases. 3. Upload: - Arduino IDE → open C:\Users\Dell\RoboGrip\arduino\arm.ino → board Uno → Upload. Open Serial Monitor 115200, send GRIP → gripper should close, UP → shoulder +5°. 4. Connect vision: Set-Location C:\Users\Dell\RoboGrip C:\Users\Dell\RoboGrip\venv\Scripts\python.exe -m pip install pyserial # check port in Device Manager → Ports (COM3/COM4...) C:\Users\Dell\RoboGrip\venv\Scripts\python.exe C:\Users\Dell\RoboGrip\src\06_serial_control.py --port COM3 Hold ✊ → terminal SENT GRIP + servo moves. Same temporal (5, 4/5) + confidence (>0.5) + E-STOP (both OPEN or e, r reset) as virtual, plus Arduino-side limits 20-160/10-150 + BLOCKED ESTOP. 5. Calibrate: if UP goes down, swap +5/-5 in .ino for that joint. Set aGr 40/90 to your gripper’s closed/open. What arm kit do you have (link/photo)? I’ll map your servo channels + angles exactly — SG90 vs MG996 + 4 vs 6 DOF changes pins/limits. |  |  |

Components required

- USB webcam (Logitech C920 / C270) or depth camera (Intel RealSense D435i)

- Laptop/PC or NVIDIA Jetson Nano/Orin Nano

- Robotic arm kit (Dobot Magician / uArm Swift) or custom-built arm with MG996R servos

- Arduino Uno/Mega or ESP32

- PCA9685 servo driver board

- Regulated power supply (5V–7.4V, multi-amp)

- Emergency-stop push-button/switch

- HC-05 Bluetooth module or Wi-Fi (via ESP32) or USB cable

- Jumper wires

- Breadboard/PCB

- Chassis/mount/3D-printed or acrylic links

- Tripod/camera mount

- LED indicators

- OLED/LCD display

- Python

- OpenCV

- MediaPipe

- scikit-learn / TensorFlow / PyTorch

- PySerial (or MQTT/socket library)

- Arduino IDE / PlatformIO

cd C:\Users\Dell\RoboGrip

.\venv\Scripts\python.exe src\02_hand_tracking.py

. \venv\Scripts\python.exe src\10_task_demo.py

THEORY